ADC of Raspberry Pi Pico

Embedded system

Assignment 1

Author: Wingren, Jesper

Examiner:Mehdi Saman Azari

Semester: 24HT

Course code: 2DT903

Exercise 1:

Answer:

We have the frequencies 1.75, 2 and 3 kHz which gives us a fmax of 3 kHz. Using the Nyquist theorem, we can decide the “best” sampling rate according to it. The Nyquist theorem is that fs  fmax * 2 = 3 * 2 = 6 kHz. Giving us a sample frequency of 6 kHz and combined with our given sampling frequency of 5 kHz we can compare using the theorem fs  6 kHz, replacing fs with 5 kHz gives us that its less leading to aliasing meaning we will not be able to perfectly reconstruct the original signal.

Exercise 2:

Answer:

To begin with a 4-bit adc includes 16 levels ranging from 0 to 15 with 0000 to 1111.

Using Step size= Vmax-Vmin16-1  and Vmax = 5, Vmin = 0.5 we get that the step size is equal to 0,3 V.

Using the midpoint 2,9V and 1000 code we construct the following diagrams.

Exercise 3:

Question 1:

Answer:

To calculate the sampling frequency, we first need to compute the conversion time using Number of cycles* clock period. We already know the number of cycles which is 96 and to calculate the clock period we take 1clock frequencythat is 48 MHZ giving us a clock period of 20.85 ns. This then gives us the conversion time of 2 μs which then is used to calculate the sampling frequency. Sampling frequency= 1conversion time=12 μs =500 kHz .

Question 2:

Answer:

The number of levels can be calculated by 212 giving it 4096 levels. Due to the pico ADC having a Vmax of 3.3 V and a Vmin of 0 V. Step size= Vmax-Vmin4096-1 0.0008 V = 0.8 mV. So, the Raspberry pi has a resolution of 12 bits meaning it can distinguish 4096 different levels of voltage.

Question 3:

Answer:

To get the voltage values and the digital values of the input analog signal a raspberry pi Pico, wires and a potentiometer was used with the given answers shown in the console.

Here is a picture of the setup followed by the code.

Below are the results printed when turning the potentiometer: